[image: image18.emf]L W H/D

A Rate for paint work

Providing and applying two coats of approved quality 

distemper paint on wall surfaces, including preparation of 

surface by cleaning, removing dust, dirt, and loose material, 

and applying one coat of primer where required to achieve a 

smooth and uniform finish, complete with all materials, 

labour, tools, and scaffolding as necessary, as per site 

instructions.

Inner side

Long wall 2 1.20 2.10 5.04

Short wall 2 0.90 2.10 3.78

celling 1 1.20 0.90 1.08

Outer side

Long wall 2 1.50 2.30 6.90

Short wall 2 1.20 2.30 5.52

Deducion

Door both side inner and outer -2 0.75 1.80 -2.70

19.62 sqm

B Rate for paint work MS ventilators

Providing and applying two coats of approved quality 

synthetic enamel paint on MS ventilators, including surface 

preparation by cleaning, removing rust, dust, and old paint 

using emery paper or wire brush, and applying one coat of 

metal primer to ensure proper adhesion and durability, 

complete with all materials, labour, tools, and scaffolding as 

required, as per site instructions.

ventilator both side 2 0.45 0.30 0.27

0.27 sqm

Dipanker Singh

SWR Pipe With Bend

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh

Unit

Item Rate 

including 

GST

Amount



S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Perpared by



G.Total

Total

Total



TENDER NOTICE
1. Background

The United Nations Sustainable Development Goal 6 (SDG 6) focuses on ensuring availability and sustainable management of water and sanitation for all. A key target (6.2) emphasizes achieving adequate and equitable sanitation and ending open defecation, with special attention to women, children, and vulnerable groups. 

In India, significant progress has been made under programs like Swachh Bharat Mission (Gramin), where most rural areas have been declared Open Defecation Free (ODF). However, field evidence shows that:

Many existing toilets are non-functional, poorly designed, or lack water supply Issues like pit filling, structural damage, poor containment, and lack of maintenance persist In several cases, toilets are constructed but not used due to usability and hygiene concerns Vulnerable households still face accessibility and dignity issues Studies highlight that availability of toilets alone is insufficient—quality, safety, and sustainability of sanitation systems are equally important. 

Objectives and Block Profile
· Upgrade non-functional or partially functional toilets 

· Improve structural stability, ventilation, and user safety 

· Ensure water availability and proper drainage

In peri-urban and rural areas like Harahua Block (Varanasi), rapid population growth, water stress, and inadequate fecal sludge management further aggravate sanitation challenges. This creates a need to upgrade (retrofit) existing sanitation infrastructure rather than only constructing new units.

Nature of Work: Construction/ retrofitting of toilets at household level in intervened Panchayats- 
Place of Work: Harhua Varanasi
GP wise interventions
	Sr. No.
	Name Of GP.
	No. of Benificiary

	1
	Palahipatti
	16

	2
	Ramgao
	24

	3
	Mohanpur
	20

	4
	Bedi
	22

	5
	Anaura
	20

	6
	Aadampur
	17

	7
	Bhatpurwa Kala
	31

	8
	Sulemapur
	10

	9
	Murdaha
	23

	10
	Gurwat
	19

	Total 
	202


	Toilet Retrofitting Detail of Components wise Quantities Block Harhua District Varanasi

	Sr. no
	Village Name 
	No. of Benificiary
	Leach Pit
	Y- Chamber
	Wall height Extension work 1 feet
	Wall height Extension work 2 feet
	Both side long Wall
	Both side short Wall
	Door
	Ventilator
	all toilet wall plaster work including inner and outer side 
	Additional wall plaster work (Repairing plaster work) -1
	Additional wall plaster work (Repairing plaster work)-2
	Slab
	Indian toilet Pan with floor
	Uropian Commode with floor
	SWR Pipe With Bend
	IEC
	Painting
	Foundation
	Leach pit Lid

	1
	Bedi
	22
	14
	15
	9
	11
	0
	4
	18
	22
	8
	7
	6
	22
	13
	1
	16
	22
	0
	0
	0

	2
	Bhatpurwa Kala
	31
	27
	29
	2
	4
	2
	5
	27
	26
	10
	2
	7
	26
	15
	0
	28
	31
	1
	1
	0

	3
	Gurwat
	19
	4
	4
	3
	0
	1
	3
	18
	16
	5
	1
	9
	17
	8
	1
	4
	19
	0
	0
	0

	4
	Mohanpur
	20
	18
	17
	10
	3
	0
	2
	18
	15
	8
	6
	2
	20
	16
	2
	17
	20
	0
	0
	1

	5
	Murdaha
	23
	11
	11
	9
	1
	1
	3
	23
	21
	5
	2
	14
	18
	7
	0
	11
	23
	0
	0
	0

	6
	Palahipatti
	16
	3
	2
	6
	2
	0
	3
	14
	10
	7
	2
	6
	10
	8
	0
	2
	16
	0
	0
	0

	7
	Ramgao
	24
	22
	16
	12
	5
	0
	1
	23
	18
	13
	4
	5
	24
	14
	0
	17
	24
	0
	0
	0

	8
	Aadampur
	17
	6
	4
	9
	1
	1
	3
	17
	15
	1
	2
	12
	16
	11
	0
	5
	17
	0
	0
	0

	9
	Anaura
	20
	12
	6
	11
	4
	1
	1
	20
	19
	3
	7
	9
	16
	10
	0
	2
	20
	0
	0
	0

	10
	Sulemapur
	10
	7
	10
	0
	4
	1
	2
	9
	9
	9
	0
	1
	10
	5
	0
	10
	10
	0
	0
	0

	Total 
	202
	124
	114
	71
	35
	7
	27
	187
	171
	69
	33
	71
	179
	107
	4
	112
	202
	1
	1
	1

	Rate As Per Estimate
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 

	Amount
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	 

	Grand Total
	 


Interested Bidders are required to submit their complete proposal along with all terms and conditions to Email id: WAIProcurementUP@wateraid.org by 27th May 2026 which should necessarily include:
For any queries pertaining to the tender, please write to on or before 25th May, 2026.
Profile of the organization, given as per the below –
2. Profile of Bidder-
1. Name of Organization:
2. Registration Certificate:
3. GST Certificate
4. MSME Certificate (if registered)
5. 3 years Tax return File (attached)
6. Address for correspondence:
7. Office Phone:
8. Mobile Phones of corresponding officials:
9. Email for correspondence:
10. Locations of operations (Head office and Branch offices):
· All certificates must be attached.
Terms and Conditions:
· Proposals
should
be
addressed
to
Jal
Seva
Charitable
Foundation,
2/92, Vishal Khand Gomti Nagar Lucknow 226010
· The Proposal must be on the company letter head with date, signature and company seal. (It is to be signed by the authorized person).
· Proposals must be submitted to Procurement Email ID:
WAIProcurementUP@wateraid.orgAny proposal submitted to any other email id will not be considered.

· The Proposal to be submitted as two separate attachment – 1. Technical proposal as per the BOQ and Financial proposal in non-editable PDF Format.
· The proposal must have the subject mentioning the village name given in the table below of point 12.
· THE PROPOSAL MUST BE SUBMITTED AS A SEPARATE ATTACHMENT FOR EACH VILLAGE
(four attachments). Kindly recommend the BOQ given.
· Proposals received after due date will not be considered.
· Financial proposals must be submitted with inclusive of applicable taxes and mentioning the validity of the quoted rate.
· All terms and conditions must be clearly mentioned in the proposals.
· Incomplete proposals shall not be considered.
· The work shall be completed on or before 20 July 2026
· Payment shall be released based on the actual work completed and verified.
3. Please provide details of two referees for the similar work done
	Description
	Referee 1
	Referee 2

	Name
	
	

	Designation
	
	

	Organization
	
	

	Description of assignment
undertaken
	
	

	Phone/ Mobile/ Email of referee
	
	


4. Last date for submission of quotation: 27/5/2026 
TENDER NOTICE
	S.
No
	Name of
the Block
	Name of the
Villages
	Landmark
	Nature of the Activity

	01
	Haruha
	Palahipatti, Ramgaon, Mohanpur, Bedi, Anura, Adampur, Bhatpurwakalan, Sulemapur,Murdaha, Gruwat
	Community
	Wall Plastering and Structural Repair



 Door Installation / Repair



Commode / Pan Replacement and Installation

 Ventilation Improvement (Ventilator)



 Pit Repair / Upgradation



Junction Chamber / Inspection Chamber


Roof Repair / Construction



 Flooring and Platform Improvement 

Wall paintings / posters promoting sanitation and hygiene (IEC)


Regards:
Procurement Team
[image: image1.emf]L W H/D

A Rate for Earthwork- Excavation

Earth work excavation (exceeding 30cm in depth, 1.5m in 

width as well as 10sqm on plan) including disposal of 

excavated soil lead upto 50m and lift upto 1.5m, disposed soil 

to be levelled and neatly dressed.

Leach Pit (πxrxrxh) 1 3.14 0.30 1.20 1.14

1.14 cum

B

Providing and fixing Precast RCC Leach Pit Cover of 1.20 

Meter Diameter

1 1.00

1.00 Nos

C Rate for Half Brick Masonary

Providing and constructing Half Brick Masonry work using 

Second Class traditional burnt clay bricks of standard size 

(230mm × 110mm × 75mm) in cement mortar 1:4 (1 cement : 

4 coarse sand), including soaking of bricks, laying in proper 

line and level, maintaining uniform joint thickness, raking of 

joints, curing, and providing necessary scaffolding, etc., 

complete in all respects as per direction of Engineer-in-

Charge.

Leach Pit wall (2xπxr)xH 1 3.14            1.20             3.77

3.77 Sq.m

Total

Perpared by



G.Total



Total



Item Rate 

including 

GST

Amount

Total



Total \

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Unit

Leach pit work

Perpared 

Date

9-Apr-26

#REF!


[image: image2.emf]L W H/D

A Rate for Plain Cement Concrete (P.C.C), 1:2:4

Providing and laying in foundation, plinth and under floors 

cement

concrete with 20mm nominal size graded stone aggregates 

(metal) i/c

ramming in layers and curing complete excluding cost of form 

work

chamber base 1 0.55 0.50 0.08 0.02

0.02 cum

B Rate for Half Brick Masonary

Providing and constructing Half Brick Masonry work using 

Second Class traditional burnt clay bricks of standard size 

(230mm × 110mm × 75mm) in cement mortar 1:4 (1 cement : 

4 coarse sand), including soaking of bricks, laying in proper 

line and level, maintaining uniform joint thickness, raking of 

joints, curing, and providing necessary scaffolding, etc., 

complete in all respects as per direction of Engineer-in-

Charge.

chamber long side wall 2 0.55            0.30             0.33

chamber short side wall 2 0.30            0.30             0.18

0.51 Sq.m

C Rate for Plastering with need cement work

Providing plastering 12 mm, thick in CM 1:6 proportion 

including scaffolding, racking out joint smooth finishing 

including curing with all lead & lift for all materials etc., 

complete

12 mm thick plaster CM 1:6

Inner side

chamber long side wall 2 0.40 0.30 0.24

chamber short side wall 2 0.30 0.30 0.18

Outer side

chamber long side wall 2 0.55 0.40 0.44

chamber short side wall 2 0.30 0.40 0.24

1.10 sqm

D

Providing and fixing granite/ besalt/ sand stone as a pit cover 

of  400mm x 450mm

1 1.00

1.00 Nos

Dipanker Singh -                       

Y Chamber

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh

Perpared by



G.Total

Total



Total

Total



Total



Total Amount

Item Rate 

including 

GST

Unit

Measurement

 Nos  DESCRIPTION OF ITEM

 Ref. As per 

market rate 

S.No.


[image: image3.emf]L W H/D

A Rate for Earthwork- Excavation

Earth work excavation (exceeding 30cm in depth, 1.5m in 

width as well as 10sqm on plan) including disposal of 

excavated soil lead upto 50m and lift upto 1.5m, disposed soil 

to be levelled and neatly dressed.

Long wall 2 1.66 0.30 0.45 0.45

Short wall 2 0.90 0.30 0.45 0.25

0.70 cum

C Rate for Brick Masonary

Providing and constructing Half Brick Masonry work using 

Second Class traditional burnt clay bricks of standard size 

(230mm × 110mm × 75mm) in cement mortar 1:6 (1 cement : 

6 coarse sand), including soaking of bricks, laying in proper 

line and level, maintaining uniform joint thickness, raking of 

joints, curing, and providing necessary scaffolding, etc., 

complete in all respects as per direction of Engineer-in-

Charge.

Long wall 2 1.66            0.23               0.65             0.50

Short wall 2 0.90            0.23               0.65             0.27

0.77 Sq.m Total



Perpared by



G.Total

Unit

Item Rate 

including 

GST

Amount

Total



\

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Toilet Foundation work

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh


[image: image4.emf]L W H/D

A Rate for Half Brick Masonary

Providing and constructing Half Brick Masonry work using 

Second Class traditional burnt clay bricks of standard size 

(230mm × 110mm × 75mm) in cement mortar 1:4 (1 cement : 

4 coarse sand), including soaking of bricks, laying in proper 

line and level, maintaining uniform joint thickness, raking of 

joints, curing, and providing necessary scaffolding, etc., 

complete in all respects as per direction of Engineer-in-

Charge.

long wall extension 2 1.50            0.30             0.90

Short wall extension 2 0.90            0.30             0.54

1.44 Sq.m

Dipanker Singh

Wall Extension

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh

Unit

Item Rate 

including 

GST

Amount

Total

Perpared by



G.Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total


[image: image5.emf]L W H/D

A Rate for Half Brick Masonary

Providing and constructing Half Brick Masonry work using 

Second Class traditional burnt clay bricks of standard size 

(230mm × 110mm × 75mm) in cement mortar 1:4 (1 cement : 

4 coarse sand), including soaking of bricks, laying in proper 

line and level, maintaining uniform joint thickness, raking of 

joints, curing, and providing necessary scaffolding, etc., 

complete in all respects as per direction of Engineer-in-

Charge.

long wall extension 2 1.50            0.60             1.80

Short wall extension 2 0.90            0.60             1.08

2.88 Sq.m

Dipanker Singh Perpared by



G.Total

Unit

Item Rate 

including 

GST

Amount

Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Wall Extension

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh


[image: image6.emf]L W H/D

A Rate for Half Brick Masonary

Providing and constructing Half Brick Masonry work using 

Second Class traditional burnt clay bricks of standard size 

(230mm × 110mm × 75mm) in cement mortar 1:4 (1 cement : 

4 coarse sand), including soaking of bricks, laying in proper 

line and level, maintaining uniform joint thickness, raking of 

joints, curing, and providing necessary scaffolding, etc., 

complete in all respects as per direction of Engineer-in-

Charge.

long wall 2 1.50            2.10             6.30

6.30 Sq.m

Dipanker Singh

Unit

Item Rate 

including 

GST

Amount

Total

Perpared by



G.Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Long wall

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh



[image: image7.emf]L W H/D

A Rate for Half Brick Masonary

Providing and constructing Half Brick Masonry work using 

Second Class traditional burnt clay bricks of standard size 

(230mm × 110mm × 75mm) in cement mortar 1:4 (1 cement : 

4 coarse sand), including soaking of bricks, laying in proper 

line and level, maintaining uniform joint thickness, raking of 

joints, curing, and providing necessary scaffolding, etc., 

complete in all respects as per direction of Engineer-in-

Charge.

Short wall 2 0.90            2.10             3.78

Deduction

Door -1 0.75            1.80             -1.35

2.43 Sq.m

Dipanker Singh

Unit

Item Rate 

including 

GST

Amount

Total

Perpared by



G.Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Short wall

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh


[image: image8.emf]L W H/D

A

Door Frame & Pannel

Providing and fixing aluminium door frame with Aluminium 

Composite Panel (ACP) cladding of approved colour and 

make, including necessary aluminium sections, screws, 

fasteners, and framing. Work also includes cutting, fixing, 

sealing with silicone, alignment, finishing, and cleaning 

complete as per drawing and site instructions. All materials, 

labour, tools & scaffolding shall be included in the rate

Door with frame size 750mm wide and 1.80 meter height 1.00

1.00 Nos

Dipanker Singh

 Ref. As per 

market rate 

DESCRIPTION OF ITEM

Door work

S.No. Unit

Women + Water collaborative Project in Varanasi Uttar Pradesh

Perpared 

Date

9-Apr-26

Measurement

Total  Nos 

Item Rate 

including 

GST

Amount

Total

Perpared by



G.Total



[image: image9.emf]L W H/D

A

MS Ventilator Providing & Fixing

Providing and fixing MS ventilator of approved size and 

design, fabricated with standard MS sections, including frame, 

louvers, hinges, and necessary fittings, fixed in position with 

proper fasteners, grouting, and alignment complete with all 

materials, labour, tools, and scaffolding as required

Ventlator size 450mm x 300mm 1.00 6.00

6.00 kg

Dipanker Singh

Amount

Total



Perpared by



G.Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

6 kg per piece

Unit

Item Rate 

including 

GST

Ventilator work

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh



[image: image10.emf]L W H/D

A Rate for Plastering work

Providing plastering 12 mm, thick in CM 1:6 proportion 

including scaffolding, racking out joint smooth finishing 

including curing with all lead & lift for all materials etc., 

complete

12 mm thick plaster CM 1:6

Inner side

Long wall 2 1.20 2.10 5.04

Short wall 2 0.90 2.10 3.78

Outer side

Long wall 2 1.50 2.30 6.90

Short wall 2 1.20 2.30 5.52

Deducion

Door both side inner and outer -2 0.75 1.80 -2.70

18.54 sqm

Dipanker Singh -                       

Total

Perpared by



G.Total

Unit

Item Rate 

including 

GST

Amount S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Inner and outer side all toilet wall plaster work

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh


[image: image11.emf]L W H/D

A Rate for Plastering work

Providing plastering 12 mm, thick in CM 1:6 proportion 

including scaffolding, racking out joint smooth finishing 

including curing with all lead & lift for all materials etc., 

complete

12 mm thick plaster CM 1:6

Inner side

Long wall 2 1.20 0.60 1.44

Short wall 2 0.90 0.60 1.08

Outer side

Long wall 2 1.50 0.60 1.80

Short wall 2 1.20 0.60 1.44

5.76 sqm

Dipanker Singh

Unit

Item Rate 

including 

GST

Amount

Total

Perpared by



G.Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Inner and outer side all toilet wall plaster work

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh


[image: image12.emf]L W H/D

A Rate for Plastering work

Providing plastering 12 mm, thick in CM 1:6 proportion 

including scaffolding, racking out joint smooth finishing 

including curing with all lead & lift for all materials etc., 

complete

12 mm thick plaster CM 1:6

Inner side

Long wall 2 1.20 0.90 2.16

Short wall 2 0.90 0.90 1.62

Outer side

Long wall 2 1.50 0.90 2.70

Short wall 1 1.20 0.90 1.08

7.56 sqm

Dipanker Singh

Inner and outer side all toilet wall plaster work

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh

Unit

Item Rate 

including 

GST

Amount

Total

Perpared by



G.Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total


[image: image13.emf]L W H/D

A

Providing and fixing sand stone/ Kota stone as a slab bottom 

of  400mm x 1200mm

4 4.00

4.00 Nos

Dipanker Singh Perpared by



G.Total

Unit

Item Rate 

including 

GST

Amount



Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Slab

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh


[image: image14.emf]L W H/D

A Indian Pan

Providing and fixing Indian type W.C. pan , white glazed 

ceramic, 580 mm size, including P-trap, embedding in cement 

concrete base, jointing with cement mortar (1:3), connection 

to soil pipe  complete in all respects as directed by the 

Engineer

1 1.00

1.00 Nos

B Rate for Plain Cement Concrete (P.C.C), 1:2:4

Providing and laying in foundation, plinth and under floors 

cement

concrete with 20mm nominal size graded stone aggregates 

(metal) i/c

ramming in layers and curing complete excluding cost of form 

work

1 1.20 0.90 0.08 0.09

0.09 cum

C Rate for Plastering with need cement work

Providing plastering 12 mm, thick in CM 1:6 proportion 

including scaffolding, racking out joint smooth finishing 

including curing with all lead & lift for all materials etc., 

complete

12 mm thick plaster CM 1:6

floor 1 1.20 0.90 1.08

1.08 sqm

Dipanker Singh Perpared by



G.Total

Unit

Item Rate 

including 

GST

Total



Amount

Total



Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Indian pan with floor

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh


[image: image15.emf]L W H/D

A Indian Pan

Providing and fixing Europian type W.C. Commode , white 

glazed ceramic,  including P-trap, embedding in cement 

concrete base, jointing with cement mortar (1:3), connection 

to soil pipe  complete in all respects as directed by the 

Engineer

1 1.00

1.00 Nos

B Rate for Plain Cement Concrete (P.C.C), 1:2:4

Providing and laying in foundation, plinth and under floors 

cement

concrete with 20mm nominal size graded stone aggregates 

(metal) i/c

ramming in layers and curing complete excluding cost of form 

work

1 1.20 0.90 0.08 0.09

0.09 cum

C Rate for Plastering with need cement work

Providing plastering 12 mm, thick in CM 1:6 proportion 

including scaffolding, racking out joint smooth finishing 

including curing with all lead & lift for all materials etc., 

complete

12 mm thick plaster CM 1:6

floor 1 1.20 0.90 1.08

1.08 sqm

Dipanker Singh

Euoropian Commode with floor

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh

Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Unit

Item Rate 

including 

GST

Amount

Total



Total

Perpared by



G.Total



Total


[image: image16.emf]L W H/D

A Rate for Earthwork- Excavation

Earth work excavation (exceeding 30cm in depth, 1.5m in 

width as well as 10sqm on plan) including disposal of 

excavated soil lead upto 50m and lift upto 1.5m, disposed soil 

to be levelled and neatly dressed.

Pipe line work 1 3.00 0.30 0.50 0.45

0.45 cum 265.50 ₹          119.48 ₹               

B Rate for Sanitary SWR work

Providing and laying SWR (Soil, Waste & Rainwater) PVC Pipe 

work of approved make and required diameter, conforming 

to relevant IS specifications, including supplying, jointing with 

rubber ring/solvent cement, fixing with necessary clamps, 

bends, tees, door tees, offsets, cowls, etc., cutting, making 

holes in walls/slabs and making good the same, testing for 

leakage, and complete installation with all labour, tools & 

plants, transportation, and incidental charges, complete in all 

respects as per direction of Engineer-in-Charge

The above scope shall be applicable for all pipes and fittings 

as listed below

110 mm dia SWR Pipe 4kg/sq.cm 3.00 Meter

110 mm dia SWR Pipe 4kg/sq.cm elbo 1.00 Nos

Dipanker Singh

SWR Pipe With Bend

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh



Perpared by



G.Total

Unit

Item Rate 

including 

GST

Amount

Total



S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total


[image: image19.png]]
WaterAid




[image: image17.emf]L W H/D

A IEC work

Providing and painting IEC branding work including logos, 

text, and designs on walls using approved quality 

enamel/acrylic paints as per approved artwork and colour 

scheme, including surface preparation, marking, 

stenciling/coating, and finishing with sharp edges and 

uniform finish complete with all materials, labour, tools and 

brushes as required as per site instructions

IEC 1 0.90 0.60 0.54

0.54 sqm

Dipanker Singh

IEC

Perpared 

Date

9-Apr-26

Women + Water collaborative Project in Varanasi Uttar Pradesh

Amount

Total

S.No.

 Ref. As per 

market rate 

DESCRIPTION OF ITEM  Nos 

Measurement

Total

Perpared by



G.Total

Unit

Item Rate 

including 

GST


